Dimensions:

| Interface |  |  |
| --- | --- | --- |
| According to |  | IEC 60169-4 |
| Electrical |  |  |
| Characteristic Impedance |  | 50 ohm |
| Frequency Range |  | DC-7.5GHz |
| VSWR |  | ≤1.10@DC-3000MHz |
| Dielectric Withstanding Voltage |  | ≥4000V RMS,50Hz,at sea level |
| Dielectric Resistance |  | ≥10000MΩ |
| Contact Resistance |  | Center Contact ≤0.4mΩ |
|  |  | Outer Contact ≤1 mΩ |
| Intermodulation 3rd (PIM3) |  | ≤ -155dBc@2×20W |
| Mechanical |  |  |
| Durability |  | Mating cycles ≥500 |
| Material and plating |  |  |
|  | Material | Plating |
| Body | Brass | Tri-Metal |
| Insulator | PTFE(Teflon) | － |
| Center conductor | Phosphor Copper | Ag |
| Gasket | Silicone Rubber | － |
| Environmental |  |  |
| Temperature Range |  | -40℃～+85℃ |
| Weatherproof Rate |  | IP 68 |
| RoHs Compliant |  |  |