1-5/8″EIA Flange Right Angle Connector For 1-5/8″Foam Feeder Cable

Interface

According to                                                                                    1-5/8In EIA Flange

Electrical

Characteristic Impedance                                                                            50 ohm

Operating Frequency Range                                                                          DC-3GHz

VSWR                                                                                                      ≤1.15 @DC-1000MHz

Dielectric Withstanding Voltage                    ≥6000V RMS,50Hz,at sea level

Dielectric Resistance                                                                                  ≥5000MΩ

Average Power                                                                                          3.4 KW@900MHz

Peak Power Maximum                                                                                            90KW

Contact Resistance                                                          Center Contact    ≤1.5mΩ

Outer Contact      ≤1.5mΩ

Mechanical

Durability                                                                                  Mating cycles ≥500

Material and plating

Material                          Plating

Body                                                Brass                                                Ni

Insulator                                          PTFE                                            －

Center conductor                            Phosphor bronze                                       Ag

Coupling Nut                                  Brass                                                            Ni

Other Parts                                      Brass                                                            Ni

Gasket                                              Silicone Rubber                                          －

Bolts and Washer                            Stainless Steel                                              －

Environmental

Operating Temperature Range                                                                -45℃～+85℃

Weatherproof Rate                                                                                              IP 67

RoHs Compliant