SMA Female Bulkhead Crimp Connector For RG58 Cable

Interface

According to                                                 IEC 60169-15

Electrical

Characteristic Impedance                                          50 ohm

Typical Frequency Range                                         DC-6GHz

VSWR                                                    ≤1.25@DC-3GHz

Dielectric Withstanding Voltage                        ≥1000V RMS,50Hz,at sea level

Dielectric Resistance                                             ≥5000MΩ

Contact Resistance                                          Center Contact ≤3mΩ

Outer Contact ≤2.5mΩ

Mechanical

Durability                                             Mating Cycles  ≥500

Material and Plating

Material             Plating

Body                         Brass                       Au

Insulator                      PTFE(Teflon)                   －

Center conductor               Phosphor Copper                     Au

Nut                          Brass                              Au

Ferrule                       Annealed Copper                     Ni

Washer                       Brass                              Au

Gasket                       Silicone Rubber                      －

Environmental

Operating Temperature Range                                  -40℃～+85℃

RoHs   Compliant