SMA Female Panel to SMA Female Adaptor

Interface

According to                                                 IEC 60169-15

Electrical

Characteristic Impedance                                          50 ohm

Typical Frequency Range                                       DC-6GHz

VSWR                                                   ≤1.10@DC-3GHz

Dielectric Withstanding Voltage                        ≥1000V RMS,50Hz,at sea level

Dielectric Resistance                                             ≥5000MΩ

Contact Resistance                                          Center Contact ≤3mΩ

Outer Contact ≤2.5mΩ

Mechanical

Durability                                               Mating Cycles ≥500

Material and Plating

Material             Plating

Body                         Brass                       Au

Insulator                      PTFE(Teflon)                  －

Center conductor               Phosphor Copper                    Au

Environmental

Operating Temperature Range                                 -40℃～+85℃

RoHs   Compliant