BNC Male to BNC Female Right Angle Adaptor

Interface

According to                                                  IEC 60169-8

Electrical

Characteristic Impedance                                          50 ohm

Frequency Range                                               DC- 4GHz

VSWR                                                ≤1.25@DC-3000MHz

Insertion Loss                                                   ≤0.10dB

Dielectric Withstanding Voltage                        ≥1500V RMS,50Hz,at sea level

Dielectric Resistance                                            ≥5000MΩ

Contact Resistance                                      Center Contact ≤1.5mΩ

Outer Contact ≤1.0 mΩ

Mechanical

Durability                                                Mating cycles ≥500

Material and Plating

Material                    Plating

Body                        Brass                               Ni

Insulator                     PTFE                             －

Center conductor              Phosphor bronze                          Au

Coupling Nut                 Brass                                     Ni

Gasket                      Silicone Rubber                             －

Environmental

Temperature Range                                              -40℃～+85℃

Weatherproof Rate                                                     IP 67

RoHs Compliant