BNC Double Male to BNC Female H Shape Adaptor

Interface

According to                                                IEC 60169-8

Electrical

Characteristic Impedance                                      75 ohm

Frequency Range                                            DC-4GHz

VSWR                                                  ≤1.30@DC-1000MHz

Dielectric Withstanding Voltage                        ≥1500V RMS,50Hz,at sea level

Dielectric Resistance                                          ≥5000MΩ

Contact Resistance                                      Center Contact ≤1.5mΩ

Outer Contact ≤1.0 mΩ

Mechanical

Durability                                                Mating cycles ≥500

Material and plating

Material                  Plating

Body                        Brass                            Ni

Insulator                     PTFE                          －

Center conductor              Phosphor bronze                        Au

Coupling Nut                 Brass                                   Ni

Other Parts                   Brass                                   Ni

Gasket                      Silicone Rubber                           －

Environmental

Temperature Range                                             -40℃～+85℃

RoHs Compliant