BNC Female Right Angle Connector For P.C.B Mount(PBT Housing)

Interface

According to                                                IEC 60169-8

Electrical

Characteristic Impedance                                       50 ohm

Frequency Range                                             DC-4GHz

VSWR                                                 ≤1.30@DC-1000MHz

Dielectric Withstanding Voltage                        ≥1500V RMS,50Hz,at sea level

Dielectric Resistance                                           ≥5000MΩ

Contact Resistance                                      Center Contact ≤1.5mΩ

Outer Contact ≤1.0 mΩ

Mechanical

Durability                                                Mating cycles ≥500

Material and plating

Material                   Plating

Body                        Zinc Alloy                           Ni

Housing                      PBT( white plastic)                  －

Insulator                      PE                              －

Center conductor               Phosphor bronze                        Au

Solder Tail                    Copper wire                              Ag

Environmental

Temperature Range                                             -40℃～+155℃

RoHs Compliant