BNC Female to BNC Female Bulkhead Adaptor

Interface

According to                                                IEC 60169-8

Electrical

Characteristic Impedance                                       75 ohm

Frequency Range                                            DC-3GHz

VSWR                                                ≤1.20@DC-3000MHz

Dielectric Withstanding Voltage                        ≥1500V RMS,50Hz,at sea level

Dielectric Resistance                                           ≥5000MΩ

Contact Resistance                                      Center Contact ≤1.5mΩ

Outer Contact ≤1.0 mΩ

Mechanical

Durability                                                Mating cycles ≥500

Material and plating

Material                  Plating

Body                        Brass                            Ni

Insulator                     PTFE                          －

Center conductor              Phosphor bronze                        Au

Coupling Nut                 Brass                                   Ni

Washer                      Brass                                   Ni

Gasket                      Silicone Rubber                           －

Environmental

Temperature Range                                             -40℃～+85℃

RoHs Compliant