BNC Male Crimp Connector For 2.5C-2V Cable

Interface

According to                                                IEC 60169-8

Electrical

Characteristic Impedance                                      75 ohm

Frequency Range                                            DC-4GHz

VSWR                                                   ≤1.30@DC-1000MHz

Dielectric Withstanding Voltage                        ≥1500V RMS,50Hz,at sea level

Dielectric Resistance                                          ≥5000MΩ

Contact Resistance                                      Center Contact ≤1.5mΩ

Outer Contact ≤1.0 mΩ

Mechanical

Durability                                               Mating cycles ≥500

Material and plating

Material                  Plating

Body                        Brass                              Ni

Insulator                     PTFE                            －

Center conductor               Brass                                 Au

Coupling bayonet              Brass                                    Ni

Crimp Ferule                 Annealed Copper                           Ni

Gasket                      Silicone Rubber                            －

Environmental

Temperature Range                                             -40℃～+85℃

RoHs Compliant