BNC Male Crimp Connector For RG179 Cable

Interface

According to                                                IEC 60169-8

Electrical

Characteristic Impedance                                       75 ohm

Frequency Range                                            DC-4GHz

VSWR                                                   ≤1.25@DC-1000MHz

Dielectric Withstanding Voltage                         ≥1500V RMS,50Hz,at sea level

Dielectric Resistance                                           ≥5000MΩ

Contact Resistance                                      Center Contact ≤1.5mΩ

Outer Contact ≤1.0 mΩ

Mechanical

Durability                                               Mating cycles  ≥500

Material and plating

Material                      Plating

Body                        Brass                              Ni

Insulator                     PTFE                            －

Center conductor              Phosphor bronze                         Au

Coupling Nut                 Brass                                    Ni

Crimp Ferule                 Annealed Copper                           Ni

Gasket                      Silicone Rubber                            －

Environmental

Temperature Range                                             -40℃～+85℃

RoHs Compliant