1.0/2.3 Male Crimp Connector For 2.5C-2V Cable

| Interface |  |  |
| --- | --- | --- |
| According to | IEC 61169-29;CECC 22230;DIN47297 |  |
| Electrical |  |  |
| Characteristic Impedance | 75 ohm |  |
| Frequency Range | DC-4GHz |  |
| VSWR | ≤1.30@DC-1GHz |  |
| Dielectric Withstanding Voltage | ≥750 V R.M.S,At sea level |  |
| Dielectric Resistance | ≥1000MΩ |  |
| Contact Resistance | Center Contact ≤4mΩ |  |
|  | Outer Contact ≤2.5mΩ |  |
| Mechanical |  |  |
| Durability | Mating cycles ≥500 |  |
| Material and plating |  |  |
|  | Material | Plating |
| Body | Brass | Ni |
| Insulator | PTFE | － |
| Center conductor | Phosphor bronze | Au |
| Resilient Contact | Phosphor bronze | Au |
| Ferrule | Annealed Copper | Ni |
| Gasket(O-ring) | Silicone Rubber | － |
| Environmental |  |  |
| Temperature Range | -40℃～+85℃ |  |
| RoHs Compliant |  |  |