FME Female Crimp Connector For RG174 Cable

| Electrical |  |  |  |  |
| --- | --- | --- | --- | --- |
| Characteristic Impedance |  |  | 50 ohm |  |
| Typical Frequency Range |  |  | DC-3GHz |  |
| VSWR |  |  | ≤1.30@DC-1GHz |  |
| Dielectric Withstanding Voltage |  |  | ≥1000V RMS,50Hz,at sea level |  |
| Working Voltage |  |  | 500V RMS,50Hz,at sea level |  |
| Dielectric Resistance |  |  | ≥5000MΩ |  |
| Contact Resistance |  |  | Center Contact ≤10mΩ |  |
|  |  |  | Outer Contact ≤5mΩ |  |
| Mechanical |  |  |  |  |
| Durability |  |  | Mating Cycles | ≥500 |
| Material and Plating |  |  |  |  |
|  |  | Material | Plating |  |
| Body |  | Brass | Ni |  |
| Insulator |  | PTFE(Teflon) | － |  |
| Center conductor |  | Phosphor Copper | Au |  |
| Ferrule |  | Annealed Copper | Ni |  |
| Environmental |  |  |  |  |
| Operating Temperature Range |  |  | -40℃～+85℃ |  |
| RoHs | Compliant |  |  |  |