1.6/5.6 Female Bulkhead Crimp Connector For BT3002 Cable

| Interface |  |  |  |
| --- | --- | --- | --- |
| According to | IEC 61169-13;CECC 22240;DIN47259 |  |  |
| Electrical |  |  |  |
| Characteristic Impedance | 75 ohm |  |  |
| Frequency Range | DC-1GHz |  |  |
| VSWR | ≤1.25@DC-1GHz |  |  |
| Dielectric Withstanding Voltage | ≥1000 V R.M.S,At sea level |  |  |
| Dielectric Resistance | ≥10000MΩ |  |  |
| Contact Resistance | Center Contact ≤4mΩ |  |  |
|  | Outer Contact ≤2mΩ |  |  |
| Mechanical |  |  |  |
| Durability | Mating cycles |  | ≥500 |
| Material and plating |  |  |  |
|  | Material | Plating |  |
| Body | Brass | Au |  |
| Insulator | PTFE | － |  |
| Center conductor | Phosphor bronze | Au |  |
| Resilient Contact | Phosphor bronze | Au |  |
| Ferrule | Annealed Copper | Ni |  |
| Gasket(O-ring) | Silicone Rubber | － |  |
| Environmental |  |  |  |
| Temperature Range | -40℃～+85℃ |  |  |
| RoHs Compliant |  |  |  |