1.6/5.6 Female Bulkhead Right Angle Clamp Connector For Flex-3Cable

Interface

According to                                 IEC 60169-13;CECC 22240;DIN47259

Electrical

Characteristic Impedance                                          75 ohm

Frequency Range                                               DC-1GHz

VSWR                                                   ≤1.05@DC-100MHz

≤1.25@100-1000MHz

Dielectric Withstanding Voltage                         ≥1000 V R.M.S,At sea level

Dielectric Resistance                                            ≥10000MΩ

Peak Working Voltage                                            330V

Contact Resistance                                       Center Contact ≤4mΩ

Outer Contact ≤2mΩ

Mechanical

Withdrawal force inner female contact                              0.5N Minimum

Withdrawal force inner male contact                                1.7N Minimum

Insertion force between female and male                             12 N Maximum

Withdrawal force between female and male                           22N Minimum

Durability                                                    Mating cycles ≥500

Material and plating

Material              Plating

Body                         Brass                       Au/Ni

Insulator                      PTFE                      －

Center conductor               Phosphor bronze                     Au

Resilient Contact               Phosphor bronze                     Au

Coupling Nut                  Brass                        Ni

Other Parts                    Brass                        Ag

Environmental

Temperature Range                                           -40℃～+85℃

RoHs Compliant