1.6/5.6 Male Straight Clamp Connector For RG59 Cable

Interface

According to                                IEC 61169-13;CECC 22240;DIN47259

Electrical

Characteristic Impedance                                         75 ohm

Frequency Range                                               DC-1GHz

VSWR                                                     ≤1.25@DC-1GHz

Dielectric Withstanding Voltage                        ≥1000 V R.M.S,At sea level

Dielectric Resistance                                            ≥10000MΩ

Contact Resistance                                      Center Contact ≤4mΩ

Outer Contact ≤2mΩ

Mechanical

Durability                                            Mating cycles  ≥500

Material and plating

Material              Plating

Body                         Brass                        Ni

Insulator                      PTFE                      －

Center conductor               Phosphor bronze                     Au

Resilient Contact               Phosphor bronze                     Au

Coupling Nut                  Brass                        Ni

Other Parts                    Brass                        Ag

Gasket(O-ring)                 Silicone Rubber                      －

Environmental

Temperature Range                                           -40℃～+85℃

RoHs Compliant