MCX Male Straight Connector For P.C.B Mount

Interface

According to                                         IEC 61169-36;CECC 22220

Electrical

Characteristic Impedance                                        50 ohm

Frequency Range                                               DC-6GHz

VSWR                                                     ≤1.10@DC-3GHz

Dielectric Withstanding Voltage                        ≥750V RMS,50Hz,at sea level

Dielectric Resistance                                            ≥1000MΩ

Contact Resistance                                     Center Contact ≤5mΩ

Outer Contact ≤2.5mΩ

Mechanical

Durability                                             Mating cycles ≥500

Material and plating

Material               Plating

Body                         Brass                         Au

Insulator                      PTFE                        －

Center conductor              Phosphor bronze                        Au

Resilient Contact               Phosphor bronze                       Au

Gasket(O-ring)                 Silicone Rubber                        －

Environmental

Temperature Range                                            -40℃～+85℃

RoHs Compliant