SMB Female (Plug) Crimp Connector For RG316 Cable Interface According to IEC 60169-10;CECC 22130;MIL-C-39012 Electrical Characteristic Impedance 50 ohm Typical Frequency Range DC-4GHz VSWR ≤1.20@DC-3GHz Dielectric Withstanding Voltage ≥1000V RMS,50Hz,at sea level Dielectric Resistance ≥1000MΩ Contact Resistance Center Contact ≤6mΩ Outer Contact ≤1mΩ Mechanical Durability Mating Cycles ≥500 Material and Plating Material Plating Body Brass Au Insulator PTFE(Teflon) － Center conductor Phosphor Copper Au Resilient Contact Phosphor bronze Au Ferrule Annealed Copper Ni (or Au) Environmental Operating Temperature Range -40℃～+85℃ RoHs Compliant