SMB Female (Plug) Right Angle Crimp Connector For RG316 Cable

Interface

According to                             IEC 60169-10;CECC 22130;MIL-C-39012

Electrical

Characteristic Impedance                                          50 ohm

Typical Frequency Range                                        DC-4GHz

VSWR                                                   ≤1.30@DC-2GHz

Dielectric Withstanding Voltage                        ≥1000V RMS,50Hz,at sea level

Dielectric Resistance                                             ≥1000MΩ

Contact Resistance                                          Center Contact ≤6mΩ

Outer Contact ≤1mΩ

Mechanical

Durability                                                Mating Cycles ≥500

Material and Plating

Material　　　　　　　　 　　　　　Plating

Body                         Brass　　　　　　                  Au

Insulator                      PTFE(Teflon)　　　　                －

Center conductor               Phosphor bronze                      Au

Resilient Contact               Phosphor bronze                      Au

Ferrule                       Annealed Copper                      Ni(or Au)

Environmental

Operating Temperature Range                                   -40℃～+85℃

RoHs   Compliant