SMB Female Crimp Connector For 2.5C-2V Cable

Interface

According to                            IEC 60169-10;CECC 22130;MIL-C-39012

Electrical

Characteristic Impedance                                          75 ohm

Typical Frequency Range                                         DC-4GHz

VSWR                                                   ≤1.30@DC-1GHz

Dielectric Withstanding Voltage                        ≥1000V RMS,50Hz,at sea level

Dielectric Resistance                                             ≥1000MΩ

Contact Resistance                                          Center Contact ≤6mΩ

Outer Contact ≤1mΩ

Mechanical

Durability                                               Mating Cycles ≥500

Material and Plating

Material              Plating

Body                         Brass                        Au

Insulator                      PTFE(Teflon)                   －

Center Conductor               Berrylium bronze                     Au

Resilient Contact               Berrylium bronze                     Au

Ferrule                       Annealed Copper                      Ni

Environmental

Operating Temperature Range                                  -40℃～+85℃

RoHs   Compliant