| SMB Female Connector For BT3002 cable( CP and Body Crimp) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Interface According to |  |  | IEC 60169-10 |  |  |
| Electrical Characteristic Impedance Frequency Range VSWR Dielectric Withstanding Voltage Dielectric Resistance Contact Resistance |  |  | 50ohm DC-3GHz ≤1.3@DC-3GHz ≥750V RMS,50Hz,at sea level ≥1000MΩ Center Contact ≤6mΩ Outer Contact ≤1mΩ |  |  |
| Mechanical Durability |  |  |  | Mating cycles≥500 |  |
| Material and plating Body Insulator Female Contact Crimp Ferrule | Material Brass PTFE Beryllium Copper Copper Alloy | Plating Au - Au Au |  |  |  |
| Environmental Temperature Range |  |  |  |  | －40℃～＋100℃ |
| Drafted by: Approved by: |  |  |  |  |  |