# SMB Female( Plug) Right Angle Connector For P.C.B Mount

Applicable Standard: MIL-C-39012;CECC-22130;IEC-80169-10 SMB type Coaxial connectors Features : 1).Cable dielectric diameter: 0.87mm(0.034-inch )or 1.5mm(0.059-inch) 2).Impedance: 50 Ohms or 75 ohms 3).Frequency range: DC up to 4GHz 4).VSWR: 1.15 to 1.35 (depends on frequency range and connector type) 6)Insulation resistance: ≥10,000M Ohms 6)Contact resistance: center contact ≤6 milliohms 7)Contact resistance: outer contact ≤1milliohms 8)Engagement force: 1.8lbs or 14.2lbs 9)Disengagement force: 1.8lbs or 14.2lbs 10)Coupling nut retention force: ≥36lbs 11)Contact captivation: ≥4lbs 12)Durability (matings): ≥500 13) Snap-on coupling per MIL-STD-348 Material and Finish: 1) Center Contact: Female : beryllium copper ,gold-plated Male: brass or beryllium copper ,gold-plated 2) Outer Contact : brass , Nickel or gold plating 3) Body ; brass , Nickel or gold plating 4) Insulator :PTFE 5) Crimp ferrule : Annealed copper alloy Applications: 1) Automotive 2) Automotive (GPS) 3) Base stations 4) Cable assembly 5) Components 6) Instrumentation 7) PC/ LAN 8) Process controls 9) Radio Boards 10) Surge Protection 11) Telecom 12) Test and Measurementa 13) Video Systems Note: 1. These characteristics are typical but may not apply to all connectors. 2. Special Specification (datasheet) for each model is base on customer’s request..