SMB Jack (Female) Straight Connector For PCB Mount Interface According to; IEC 60169-10 CECC 22130. Electrical Characteristic Impedance Frequency Range VSWR Dielectric Withstanding Voltage Dielectric Resistance Contact Resistance; 50ohm DC-4GHz ≤1.2@DC-3GHz ≥1000V RMS,50Hz,at sea level ≥1000MΩ Center Contact≤6mΩ Outer Contact≤1mΩ. Mechanical Durability; Mating cycles≥500. Material and plating Body Insulator Female Contact; Material Brass PTFE Phosphor Copper; Plating Au - Au. Environmental Temperature Range; －40℃～＋125℃. Drafted by: Approved by:..