SMB Male( Solder Pin)Plug Crimp Connector For RG174 Cable

Interface

According to                            IEC 60169-10;CECC 22130;MIL-C-39012

Electrical

Characteristic Impedance                                          50 ohm

Typical Frequency Range                                        DC-4GHz

VSWR                                                   ≤1.25@DC-1GHz

Dielectric Withstanding Voltage                        ≥1000V RMS,50Hz,at sea level

Dielectric Resistance                                             ≥1000MΩ

Contact Resistance                                          Center Contact ≤6mΩ

Outer Contact ≤1mΩ

Mechanical

Durability                                                Mating Cycles ≥500

Material and Plating

Material              Plating

Body                         Brass                       Au

Insulator                      PTFE(Teflon)                  －

Center conductor               Phosphor Copper                    Au

Resilient Contact               Phosphor bronze                     Au

Gasket(O-ring)                 Silicone Rubber                     －

Ferrule                       Annealed Copper                    Au

Environmental

Operating Temperature Range                                 -40℃～+85℃

RoHs   Compliant