SMC Female Right Angle Crimp Connector For RG58 Cable

| Interface |  |  |  |
| --- | --- | --- | --- |
| According to |  | IEC 60169-9；CECC22140；DIN47225 |  |
| Electrical |  |  |  |
| Characteristic Impedance |  | 50 ohm |  |
| Typical Frequency Range |  | DC-6GHz |  |
| VSWR |  | ≤1.25@DC-3GHz |  |
| Dielectric Withstanding Voltage |  | ≥1000V RMS,50Hz,at sea level |  |
| Dielectric Resistance |  | ≥5000MΩ |  |
| Contact Resistance |  | Center Contact ≤6mΩ |  |
|  |  | Outer Contact ≤2.5mΩ |  |
| Mechanical |  |  |  |
| Durability |  | Mating Cycles ≥500 |  |
| Material and Plating |  |  |  |
|  |  | Material | Plating |
| Body |  | Brass | Au |
| Insulator |  | PTFE(Teflon) | － |
| Center conductor |  | Phosphor Copper | Au |
| Resilient Contact |  | Phosphor bronze | Au |
| Ferrule |  | Annealed Copper | Ni |
| Gasket(O-ring) |  | Silicone Rubber | － |
| Environmental |  |  |  |
| Operating Temperature Range |  | -40℃～+85℃ |  |
| RoHs | Compliant |  |  |