SMC Male Right Angle Connector For P.C.B Mount

Interface

According to                             IEC 60169-9；CECC22140；DIN47225

Electrical

Characteristic Impedance                                          50 ohm

Typical Frequency Range                                         DC-6GHz

VSWR                                                   ≤1.20@DC-3GHz

Dielectric Withstanding Voltage                        ≥1000V RMS,50Hz,at sea level

Dielectric Resistance                                             ≥5000MΩ

Contact Resistance                                          Center Contact ≤6mΩ

Outer Contact ≤2.5mΩ

Mechanical

Durability                                               Mating Cycles ≥500

Material and Plating

Material              Plating

Body                         Brass                        Au

Insulator                      PTFE(Teflon)                    －

Center conductor               Phosphor Copper                      Au

Resilient Contact               Phosphor bronze                      Au

Environmental

Operating Temperature Range                                 -40℃～+85℃

RoHs   Compliant