SMZ(BT43) Female(Solder Pin) Crimp Connector For BT3002 Cable

Interface

According to                                               BS9210F0022

Electrical

Characteristic Impedance                                         75 ohm

Frequency Range                                               DC-3GHz

VSWR                                                  ≤1.30@DC-1GHz

Dielectric Withstanding Voltage                    ≥1500V RMS,50Hz,at sea level

Dielectric Resistance                                            ≥5000MΩ

Contact Resistance                                    Center Contact ≤5mΩ

Outer Contact ≤1mΩ

Mechanical

Durability                                             Mating cycles ≥250

Material and plating

Material              Plating

Body                         Brass                        Ni

Insulator                      PTFE                      －

Center conductor               Beryllium bronze                     Au

Spring Contact                 Phosphor bronze                     Au

Ferrule                       Annealed Copper                     Ni

Environmental

Temperature Range                                           -40℃～+100℃

RoHs Compliant