UHF Female to UHF Female Bulkhead Adaptor

Interface

According to                                               IEC 60169-12

Electrical

Characteristic Impedance                                   Non-Constant

Frequency Range                                              DC-300MHz

VSWR                                                    ≤1.20 @DC-300MHz

Dielectric Withstanding Voltage                    ≥2000V RMS,50Hz,at sea level

Dielectric Resistance                                        ≥5000MΩ

Contact Resistance                                 Center Contact ≤5.0mΩ

Outer Contact ≤5.0mΩ

Mechanical

Durability                                            Mating cycles ≥500

Material and plating

Material              Plating

Body                         Brass                         Ni

Insulator                    PTFE                        －

Center conductor             Phosphor bronze                   Ag

Coupling Nut                 Brass                               Ni

Environmental

Temperature Range                                           -40℃～+85℃

Weatherproof Rate                                               IP 67

RoHs Compliant