TNC Male Right Angle Clamp Connector For LMR400 Cable(B style)

Interface

According to                                        IEC 60169-17;CECC 22200

Electrical

Characteristic Impedance                                        50 ohm

Frequency Range                                              DC-11GHz

VSWR                                                    ≤1.25@DC-3000MHz

Dielectric Withstanding Voltage                        ≥1500V RMS,50Hz,at sea level

Dielectric Resistance                                            ≥5000MΩ

Contact Resistance                                      Center Contact ≤10mΩ

Outer Contact ≤2.5 mΩ

Mechanical

Durability                                                Mating cycles ≥500

Material and plating

Material                  Plating

Body                        Brass                              Ni

Insulator                     PTFE                            －

Center conductor              Phosphor bronze                         Au

Resilient Contact              Phosphor bronze                           Au

Couping Nut                  Brass                              Ni

Other Parts                   Brass                              Ag

Gasket                      Silicone Rubber                            －

Environmental

Temperature Range                                             -40℃～+85℃

Weatherproof Rate                                               IP 67

RoHs Compliant